Smart Parking Lot System Documentation: Simple Overview

Tired of driving around in circles looking for an open spot? The Smart Parking Lot system is designed to solve this common problem by using automation to manage parking spaces.

This system uses small electronic brains (microcontrollers) and tiny digital eyes (sensors) to instantly detect which parking spots are open and which are taken. It manages all parking slots efficiently and shows the availability to drivers in real-time, making the parking process much faster and less stressful.

II. Team Members

The dedicated team behind this project includes:

- S.S.S. Prabhakar

- MOHD. Naziya

- V. Srujana

- B. Nithya

- A. Vikram Teja

- Sana Mehreen

- V. Nakshatra

- Shaik. Akhil

- CH. Takur

III. Components Used: The System's Building Blocks

The project prototype is built using simple, low-cost hardware:

- Microcontroller (The Brain): Arduino UNO

  - This is the main control unit that reads the sensor data and controls the status lights.

- Sensors (The Eyes): Ultrasonic/IR Sensors

  - These devices are placed in each parking spot and detect the presence of a vehicle (or lack thereof) by measuring distance or reflecting light.

- Indicators (The Signals): LEDs (Red/Green)

  - Simple lights used to visually show the status of the parking slot right where the driver needs it.

- LCD Display
Used to display the output of the Arduino.

- Other Essentials: Jumper Wires (for connections), Power Supply, and a Bread Board (for prototyping).

IV. How It Works: Working Principle and Architecture

The system works in three simple, automated steps:

Working Principle

- Checking the Slot (Detection):

  - A vehicle pulls into a spot, and the Sensor detects its presence, acting like a digital tripwire.

- The Brain Thinks (Processing):

  - The sensor sends a signal to the Arduino UNO (the brain). The Arduino instantly processes this signal to determine if the slot is occupied or empty.

- Showing the Status (Indication):

  - Based on the Arduino's decision, it lights up an LED:

    - Green: Means the slot is Empty (Available).

    - Red: Means the slot is Occupied (Unavailable).

- Guiding the Driver (Monitoring - Optional):

  - This real-time information can also be sent to a main Display screen at the lot's entrance or a mobile application, guiding the driver directly to the nearest open spot without circling.

System Architecture

The basic flow of data in the system is: Sensor → Arduino (Processing) → LED Indicator → (Optional Display/Server for Driver Guidance)

V. Key Advantages (Why Smart Parking is Better)

This system provides several benefits for both drivers and parking lot operators:

- Saves Time and Reduces Congestion: Drivers can find open spots much faster, meaning less time spent driving around the parking lot. This also reduces traffic jams inside and near the facility.

- Improves Efficiency: The system helps the parking lot make the best use of every single space.

- Low-Cost and Scalable: Since it uses simple and affordable components like the Arduino and standard sensors, it can be built and easily expanded to fit any size, from a small office lot to a massive multi-level parking garage.

VI. Applications (Where It Can Be Used)

The Smart Parking Lot system is perfect for any location where efficient parking management is needed:

- Malls and Shopping Centres: To quickly guide visitors to open spaces during peak hours.

- Office Buildings: To manage employee and visitor parking efficiently.

- Apartment Complexes: To monitor and allocate residential parking spaces.

- Public Parking Lots: Such as those near train stations, airports, or downtown areas.

VII. Conclusion

The Smart Parking System uses simple automation to deliver efficient vehicle management. It's a practical, modern solution that is easy to build using accessible components like the Arduino and standard sensors, proving that smart solutions don't have to be complicated.

Arduino Code:

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// Tinkercad I2C LCD address = 0x27

LiquidCrystal_I2C lcd(0x27, 16, 2);

// IR sensor pins

int ir1 = 2;

int ir2 = 3;

int ir3 = 4;

// LEDs for each slot

// Slot 1

int green1 = 7;

int red1   = 8;

// Slot 2

int green2 = 9;

int red2   = 10;

// Slot 3

int green3 = 11;

int red3   = 12;

void setup() {

lcd.init();

lcd.backlight();

// IR sensor inputs

pinMode(ir1, INPUT);

pinMode(ir2, INPUT);

pinMode(ir3, INPUT);

// LED outputs

pinMode(green1, OUTPUT);

pinMode(red1, OUTPUT);

pinMode(green2, OUTPUT);

pinMode(red2, OUTPUT);

pinMode(green3, OUTPUT);

pinMode(red3, OUTPUT);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(" SMART PARKING ");

delay(2000);

}

void loop() {

int s1 = digitalRead(ir1); // 1 = free, 0 = occupied

int s2 = digitalRead(ir2);

int s3 = digitalRead(ir3);

// LED Logic

// Slot 1

if (s1 == 1) {

digitalWrite(green1, HIGH);

digitalWrite(red1, LOW);

} else {

digitalWrite(green1, LOW);

digitalWrite(red1, HIGH);

}

// Slot 2

if (s2 == 1) {

digitalWrite(green2, HIGH);

digitalWrite(red2, LOW);

} else {

digitalWrite(green2, LOW);

digitalWrite(red2, HIGH);

}

// Slot 3

if (s3 == 1) {

digitalWrite(green3, HIGH);

digitalWrite(red3, LOW);

} else {

digitalWrite(green3, LOW);

digitalWrite(red3, HIGH);

}

// Count free slots

int free_slots = s1 + s2 + s3;

// Display on LCD

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Free Slots: ");

lcd.print(free_slots);

lcd.setCursor(0, 1);

lcd.print("1:");

lcd.print(s1 ? "F " : "O ");

lcd.print("2:");

lcd.print(s2 ? "F " : "O ");

lcd.print("3:");

lcd.print(s3 ? "F " : "O ");

delay(1000);

}